CPE301 – SPRING 2019

# Design Assignment 2A

Student Name: Victor Martinez

Student #:5003131674

Student Email: martiv6@unlv.nevada.edu

Primary Github address: https://github.com/martiv6

Directory: https://github.com/martiv6/submissions_da/tree/master/DesignAssignment/DA2

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

LED

TASK 1

SHIELD

328P

Mini

POWERSUPPLY

TASK 2

LED

SWITCH

SHIELD

328P

Mini

POWERSUPPLY

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

;

; 2A_task_1.asm

;

; Created: 3/1/2019 8:18:16 PM

; Author : victor

;

.org 0

LDI		r16, 0b0111		; setting pd2

OUT		DDRB, r16		; enableing pd2 as output

LDI		r17, 0b0000		; used to set/reset pd5

LDI		r20, 0b0101		; setting up TCCR1B = 00000101 for 1024

STS		TCCR1B,	r20		; storing the value in TCCR1B

INTRO:

LDI		r20, 0b0000		; setting up timer counter to 0

STS		TCNT1H, r20 	; Timer counter is 16 bits

STS		TCNT1L, r20 	; Thus, requires 2 8 bit regs

RJMP	SLOWDOWN 		; jumps back to SLOWDOWN

COMEBACK:

EOR		r17, r16		; xor to toggle LED

OUT		PORTB, r17		; enable pd5

LDI		r20, 0b0000		; setting up timer counter to 0

STS		TCNT1H, r20 	; Timer counter is 16 bits

STS		TCNT1L, r20 	; Thus, requires 2 8 bit regs

RJMP	FINAL_STOP		; Jumps to FINAL_STOP

COMEBACK_DOS:

EOR		r17, r16		; xor to toggle LED

OUT		PORTB, r17		; enable pd5

RJMP	INTRO			; repeat main loop

SHOWDOWN_1:

CPI		r29, 0x11		; check if upper timer counter have reached desired value

BRLT	SLOWDOWN		; otherwise recheck the upper bytes

RJMP	COMEBACK		; jumps to COMEBACK

SLOWDOWN:

LDS		r29, TCNT1H		; load upper bytes of timer counter to r29

LDS		r28, TCNT1L 	; load lower bytes of timer counter to r28

CPI		r28, 0xB2		; check to see if lower 8 bits of timer counter

BRSH	SHOWDOWN_1		; if lower bit not

RJMP	SLOWDOWN		; otherwise keep checking lower bytes

SHOWDOWN_2:

CPI		r29, 0x1A		; check if upper timer counter have reached desired value

BRLT	FINAL_STOP		; otherwise recheck the lower bytes

RJMP	COMEBACK_DOS	; jumps to COMEBACK_DOS

FINAL_STOP:

LDS		r29, TCNT1H		; load upper bytes of timer counter to r29

LDS		r28, TCNT1L 	; load lower bytes of timer counter to r28

CPI		r28, 0x8C		; check to see if lower 8 bits of timer counter are 0x08

BRSH	SHOWDOWN_2		; keep checking lower bytes

RJMP	FINAL_STOP		; jumps to FINAL_STOP

/*

* 2A_Task_1_C.c

*

* Created: 3/1/2019 12:09:53 AM

* Author : victor

*/

#include <avr/io.h>

#define F_CPU 16000000UL

#include <util/delay.h>

int main(void)

{

DDRB = 0b0100;				//makes pd2 an output

while (1)

{

PORTB = (1 << PORTB2);	//turns on the led we want

_delay_ms(435);		//time the led will be off to get the 60% DC with the period of 0.725

PORTB= ~(1 << PORTB2);	//will turn on led

_delay_ms(290);		//the time the led will be on to get the 40% DC with the period of 0.725

}

return 0;

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

;

; 2A_Task2_asm.asm

;

; Created: 3/2/2019 12:20:24 PM

; Author : victor

;

.org 0

SBI	DDRB, 0b0010		; makes PB2 the output

CBI	DDRC, 0b0000		; makes A1/S1 the button to  push

SBI	PORTB, 0b0010		; makes PB2 turn off

THEWAIT:

SBIC	PINC, 0b0001	; if pressed do this

RJMP	THEWAIT			; if not pressed jump back to THEWAIT

RCALL	THEPUSH			; keeps led to 1.25s

SLOWDOWN:

LDS	R29, TCNT1H			; load upper bytes of timer counter to r29

LDS	R28, TCNT1L 		; load lower bytes of timer counter to r28

CPI	R28, 0x4A			; check to see if lower 8 bits of timer counter are valid

BRSH	WELP			; if same or higher if not goes back to welp

RJMP	SLOWDOWN		; otherwise keep checking lower bytes

THEPUSH:

CBI		PORTB, 0b0010	; if pressed it will light up the led

LDI		R20, 0b0101	    ; set the prescalar to 1024

STS		TCCR1B, R20		; loads value to R20

LDI		R20, 0b0000		; resets timer

STS		TCNT1H, R20		; clears value in TCNT1H

STS		TCNT1L, R20 	; clears value in TCNT1L

RJMP	SLOWDOWN		; jumos down to SLOWDOWN to start again

COMINGHOME:

RJMP	THEWAIT			; goes to back to see in theres an input

SBI		PORTB, 0b0010	; if on display on LED

RJMP	THEWAIT			; goes back to see if there is an input

WELP:

CPI	R29, 0x4C			; check if upper timer counter have reached desired value

BRLT	SLOWDOWN		; otherwise recheck the lower bytes

SBI	PORTB, 0b0010		; TURN OFF LED

RET

/*

* 2A_Task_2_C.c

*

* Created: 3/2/2019 6:16:17 PM

* Author : victor

*/

#include <avr/io.h>

#define F_CPU   16000000UL

#include <util/delay.h>

int main(void)

{

DDRB = 0b0100;			// set PB2 as output when pressed

PORTB = (0b0001<<0b0101);		// turn off PB5 LED

DDRC = (0b0000<<0b0001);		// reads data entered

PORTC = (0b0001<<0b0001);		// will cause it to light up and display

while (1)

{

if(PINC & (PINC1<<PINC1))	//to see if the switch is pressed

{

PORTB = (PORTB1<<PORTB2);	// if there is no input keep led off

}

else

{

PORTB &= ~(PORTB1<<PORTB2);	// toggle the led

_delay_ms(1250);	// keep the light on for 1.25 SECONDS

}

}

return 0;

}

- SCHEMATICS

TASK 1.

TASK 2.

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

TASK 1&2

- VIDEO LINKS OF EACH DEMO

TASK 1 – https://youtu.be/C1VlHoTSl8A

C- https://youtu.be/LN9se80oZAE

TASK 2 - https://youtu.be/xiePFdgSiS8

C- https://youtu.be/eeDtTQRLvYc

- GITHUB LINK OF THIS DA

https://github.com/martiv6/submissions_da/tree/master/DesignAssignment/DA2

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Victor Martinez