CPE301 – SPRING 2019

# Design Assignment 4

Student Name: Victor Martinez

Student #: 5003131674

Student Email: martiv6@unlv.nevada.edu

Primary Github address: https://github.com/martiv6

Directory: https://github.com/martiv6/submissions_da/tree/master/DesignAssignment/DA4

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

DC Motor

MD08A

(Driver)

BREAD

BOARD

328P

Mini

POWERSUPPLY

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* DA4A.c

*

* Created: 4/10/2019 10:48:31 AM

* Author : victor

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

void adc_initializer(void);												// void function we will use for ADC

int motor_control;														// to be able to control the motor with the potentiometer

int	motor_status = 0;													// to toggle the motor with the switch will be off at first

int main()

{

DDRD = 0b01000000;													// sets port d as output

DDRC = 0b0010;														// sets port c as output

PORTC |= (0b0001<<0b0001);											// enable pull-up pin

PCICR = 0b0010;														// enable PCIE1 to make PCMSK1 work

PCMSK1 = 0b0010;													// enable pin changes

TCCR0A= 0b10000011;													// set fast PWM and clears OCR0A on MATCH

TCCR0B= 0b0101;														// set prescaler to 1024

sei();																// enable interrupts

adc_initializer();													// initializes ADC

while (1)

{																	// waits for an interrupt

}

}

ISR(PCINT1_vect)

{

if(!(PINC & (1<<PINC1)))											// when the button is pressed it goes into this loop to determine if will be on or off

{

if(motor_status == 0)											// when the button is pressed the motor will stop

{

OCR0A = 0;													// stops the motor

_delay_ms(1500);											// meant for debouncing to make sure its a clean reading

}

if (motor_status == 1)											// when the button is pressed again it will stay on

{

while((ADCSRA&(1<<ADIF))==0);								// wait for conversion

motor_control = ADC;										// puts ADC value into motor control so we can then keep it on

OCR0A = motor_control;										// Output to converted value to 0CR0A and makes the motor run

_delay_ms(1500);											// meant for debouncing to make sure its a clean reading

}

motor_status ^= 1;												// update state of motor to on

}

}

void adc_initializer(void) 												//initialize ADC

{

ADMUX = (0<<REFS1);													// ADC ENable

ADCSRA = (1<<ADEN)| (1<<ADSC)| (1<<ADATE)| (0<<ADIF)|				// ADC Start Conversion

(0<<ADIE)| (1<<ADPS2)|(1<<ADPS1)| (1<<ADPS0);						// ADC Auto Trigger Enable

// ADC Interrupt Flag

// ADC Interrupt Enable

// ADC Prescaler Select Bits

// ADC Prescaler select bits

// ADC input

// Timer/Counter1 Interrupt Mask Register

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

N/A

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

N/A

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/Bv9w_7TFTfo

- GITHUB LINK OF THIS DA

https://github.com/martiv6/submissions_da/tree/master/DesignAssignment/DA4

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

VICTOR MARTINEZ